Resilient Routing in Wireless Mesh Networks using Digital Twin and GNN-Based Reinforcement Learning

Anandakrishnan K V, Drupad M D

Department of Computer Science and IT, School of Computing, Amrita Vishwa Vidyapeetham, Kochi, Kerala, India

Abstract – A Wireless Mesh Network (WMN) is a communications network made up of multiple radio nodes organized in a mesh topology used for connecting dynamically with each other. Their flexibility in dynamic connectivity and multi-hop capabilities can attract major threats. Particularly, jamming attacks can severely degrade network performance by disrupting routing and data transmission. Traditional routing protocols lack the adaptability to operate effectively in such dynamic and hostile environments. This article proposes an integrated resilience framework for WMNs that combine digital twin modelling, graph neural networks, and reinforcement learning. A digital twin simulates realistic network behaviour and jamming scenarios, while a GAT model captures the topological and traffic features. A multi-agent deep Q-network utilizes these embeddings to learn adaptive routing strategies under varying jamming conditions.

Index Terms – Digital twin, routing, jamming detection, DQN, GNN.

- Introduction

Wireless communication networks have greatly influenced the advancement of interconnectivity of the technology without a physical medium. Main problem is that the reach of a single wireless device is far less to be suitable for a proper networking. This is where Wireless Mesh Networks (WMNs) fill the gap. WMNs use multi-hop routing. This extends the coverage of data transmission. Multiple nodes are interconnected and take part in a single information transfer. Having a decentralized architecture makes them handy in real-world applications like disaster response, industrial automation, and Internet-of-Things (IoTs). Due to this flexibility, WMNs are also prone to different kind of attacks. These attacks damage the interconnectivity of the nodes, affect network reliability, delays sensitive communication and affect overall Quality-of-Service.

Conventional methods for network resilience, includes heuristic-based topology control and rule-based routing. These fall short in dynamic environments. Whereas, the mathematical optimization approaches face scalability limitations. Reinforcement Learning (RL) techniques have shown promise in adaptive, real-time decision-making in dynamic environments. However, many such methods still rely on centralized architectures degrades the decentralized nature of WMNs.

To mitigate these problems, this paper proposed a novel approach that integrates three technologies including Digital Twin (DT), Multi-Agent Deep Q-Networks (DQN) and GAT. These three together forms the basis for jamming detection and fault tolerance in resilient WMNs.

Let us now look into these technologies briefly.

A Digital Twin (DT) is like a real-time virtual mirror of the real network which helps in the detection of anomalies and analysis of predictive failures. Comparison between the actual and expected radio signals, they are able to detect jamming attacks and analyse their impacts.

Multi-Agent DQN helps to extend traditional reinforcement learning by enabling the nodes to collaboratively adapt their routing and transmission policies. Each node has an agent embedded with it. The agent learns to optimize local and global performance using its own policy network while sharing experiences across agents.

Graph Attention Network (GAT) is an attention-based Graph Neural Network (GNN) framework. They are used to extract structural features from the network graph. By aggregating information from neighbouring nodes through learned attention weights, GATs can generate node embeddings which helps in guiding routing decisions. This can also facilitate the detection of anomalies by understanding the global topology.

In short, these components, can detect and mitigate threats, and also adapt to the changing network conditions. This will form the basis for a scalable and dynamic routing solution for WMNs.

- Related Works

Recent research in Wireless Mesh Networks (WMNs) have shifted the focus from heuristic protocols to machine learning-oriented approaches. This is mostly driven by the need to handle increasingly complex network dynamics such as mobility of nodes, spectral interference, and dynamic traffic loads. In short, make the whole system dynamic.

Routing using Reinforcement Learning and Graph Neural Networks (GNNs):

Routing in Wireless Mesh Networks (WMNs) is typically modeled as a Markov Decision Process (MDP). This framework allows autonomous agents to develop optimal routing policies by exploring and interacting with their environment.

Deep Reinforcement Learning (DRL) and GNNs: Pappone et al. (2023) introduced a framework that integrates GNNs with Multi-Agent-DRL to overcome a core weakness in traditional Reinforcement Learning: the inability to generalize to unfamiliar network topologies without significant retraining. By leveraging GNNs to encode the network state, their model captures the graph's permutation-invariant structure, enabling agents to minimize flow collisions and adapt to real-time changes.

Resilient Emergency Routing: Jin et al. (2025) proposed Deep Reinforcement Learning-based Resilient Routing (DRLRR) specifically for urban emergency communication networks. Their approach utilizes Double Deep Q-Networks (DDQN) to mitigate the overestimation bias common in standard Q-learning. This strategy accurately captures dynamic topology changes by optimizing message formats and integrating node and link state information into a specialized reward function, ensuring stable data transmission during disasters.

- Methods

Now let us dive deep into the proposed routing framework. It integrates a Digital Twin based network model with graph-based Reinforcement Learning for resilience in routing. It helps it to be intelligent, also makes it adaptive to the network conditions.

  - Digital Twin

This framework is based on a Digital Twin (DT), which is implemented through a Mesh Digital-Twin module. Conventional routing mechanisms depend on packet-level feedback but, the Digital Twin maintains a virtual copy of the physical network. This virtual representation is continuously updated.

Digital Twin includes the following parts:

Radio Map: The Digital Twin takes in environmental radio maps containing Received Signal Strength (RSS) measurements. These values are linked to the link-level performance indicators. They include latency and bandwidth capacity. Sigmoidal transformation functions are used to convert the RSS values into estimates. This will allow the model to understand non-linear signal propagation effects. This helps the DT to show the real-time variations in channel conditions as well as the interference in the environment.

Feature Enrichment and Anomaly Awareness: To improve the environmental awareness, anomaly detection and jamming analysis modules are used by DT. These include AnomalyBridge and JammerDetector. These components improve upon the network graph by including additional information into the node’s representations. Each node has a seven-dimensional feature vector to represent it. It consists of average latency, average bandwidth, anomaly scores, estimated jamming probabilities average jamming intensity, and routing role indicators.

These features can give the agent a better understanding of the network as well as the environmental challenges it could face. This can also help the agent mitigate previously unknown disruptions.

  - Graph Attention Networks

For the proposed framework, the decision making is implemented using a Graph Attention Network (GAT). It is used to create an embedding of the entire topology of the network and the dependencies between each node.

Input representation: The seven-dimensional feature vector that was referred to earlier in Section 3.1 is given as the input to the GAT. Graph-based formulation of GAT helps the model to learn about both the topography and environmental information.

Message Passing: Information is passed between neighbour nodes using two Graph Attention Network layers. Using attention, the nodes can focus on important, close-by neighbours based on their relevance to routing. This helps to pass information about node failures or jamming attacks across the network. This will affect the neighbour nodes’ representations.

Q-Value Estimation: The output embeddings generated by the attention layers are passed through fully connected layers to estimate the Q-values. Each Q-value denotes the reward that the agent can get by selecting each neighbour as the next hop. In this, routing decision are taken as discrete actions, and for each action there will be a corresponding reward.

  - Deep Q-Network Learning

The learning process of the models is based on the Deep Q-Network (DQN) architecture which is adapted for graph-like environments such as mesh networks. The agent combines the GAT's state representations with Reinforcement Learning techniques. Some parts of this learning are:

Experience replay: While undergoing training, interaction tuples s,a,r,s' are stored in a replay memory. Subsets from these batches are randomly sampled from this memory. This will reduce temporal correlations between the training samples themselves. This improves the convergence stability.

Exploration Strategy: The framework uses an epsilon-greedy policy. This balances exploration and exploitation. In the beginning, the agent explores its options in the environment. Later in the training phase, the exploration rate ε decays gradually. This will make the agent rely on the learned policies.

  - Rewarding Function

The framework uses rewards based on the agent’s routing decision. It promotes efficient routing and minimizes detours.

Base reward: Base reward is calculated according to the agent’s progress towards the destination. It promotes less-hop more-reward policy.

Anomaly penalization: The agent gets penalized when it routes through unstable nodes. Anomaly dampening factor: 
R = Rbase ×  1 - α × AnomalyScore

Where, α controls the impact of detected anomalies. Nodes that show abnormal behaviour gets less rewards.

Jamming penalty: A negative reward is given when the agent selects a node that is identified as jammed. This prevents the routing through highly interfered regions.

Resilience Incentive: If the routing is completed successfully an additional reward is given. This stresses the need for a complete routing, making the whole network resilient in nature.